How to create a fake ECU for YC600 inverters

(unofficial)

Guide V1.0+

1.	Get Z Stack 3.0.2 here:	1

2.	Get the script file and the binary for CC2531:	1

3.	How to flash CC2531	1

4.	How to use Z-TOOL	2

5.	Use provided scripts	5

## Get Z Stack 3.0.2 here:

Z-STACK A fully compliant ZigBee 3.x solution: Z-Stack | TI.com

## Get the script file and the binary for CC2531:

## How to flash CC2531

Plug CC2531 Stick into USB Port and plug in CC Debugger into USB Port check wiring pin 1 must be always one. If the LED of CC Debugger Illuminates RED you have a wrong wiring. GREEN means everything is OK.

Start “Flash Programmer” it is from TI.

C:\Program Files (x86)\Texas Instruments\SmartRF Tools\Flash Programmer\bin

Select the provided hex file (CC2531ZNP-with-SBL.hex).

Sometimes you need to disable “retain IEEE address” option to get it work.

If you want to go back to original TI Sniff Firmware choose this path:

C:\Program Files (x86)\Texas Instruments\SmartRF Tools\Packet Sniffer\bin\general\firmware

If this is done unplug the CC Debugger and remove the CC2531 and plug it back into the USB port.

Then wait that the green LED TURNS OFF !!!! sometimes you have to wait at least 2 minutes. If you do not wait, Z-tool will not find your stick.

## How to use Z-TOOL

Go to  and start “Z-TOOL” it needs administrator rights.

C:\Texas Instruments\Z-Stack 3.0.2\Tools\Z-Tool

If it is started check version Help  About

Then you should see your previous flashed CC2531 and a COM Port.

In this case it is COM port 10 can be different in your case.

Most important thing is to check the COM port configuration, go to: TOOLSSettingsserial Devices

Then double click on ComXX setting should be like this. COM port can be different.

## Use provided scripts

Go to TOOLSscript window

Go to file and load the provided scripts.

Before you start the scripts you have to modify them but you don’t have to reload them. Use your favorite editor or notepad++. Make sure you save the scripts as text after editing them.

1. Script coordinator_ECU_pair.zjs

This one has to be run for every inverter you have in order to “pair” them with the fake ECU. Every time you pair an inverter, make sure you edit the script to include the serial number of the inverter you want to pair. Your INVERTER serial number it is written on the inverter or can be read out of the network link state telegram. This is periodically sent during the daylight (ZigBee unit in inverter is powered by sun).

The number looks like the value of INV_1_SN (408000066781) in the script. Replace this number  with your serial number.

Also, adjust the COM port accordingly. Make sure everything is set correctly. There is not error correction whatsoever, so if you make a mistake (e.g. typo) pairing will not work.

Save and run pairing script once. Running the scripts is done in the script window of Z-Tool:

File → Load → Select script. When loaded, select script and click Run.

Check the log (trace.log) to see if there are errors. SAVE(!) the inverter address (e.g. 0xc28c) from the log, you will need it for the 2nd script.

This address can also be obtained from the link state telegram, as shown above with red (e.g. 0x0C11).

If pairing is done correctly Inverter is ready to sends data to the PAN ID shared with the fake ECU.

2. coordinator_ECU_normal.zjs

This script can be used after pairing the inverters to collect data and log it to a file (Trace.log).

Before you ca use this script, you need to change the COM port and add the serial numbers and addresses of all the inverters you want to read out.

Also adjust the number of inverters (INV_NUM) you want to read out.

Make sure you make no mistakes here otherwise the script will not work.

Optionally, you may tailor the script execution by adjusting some of the other parameters:

If set true, this script will only listen and log data of paired inverters when it is requested by a real ECU. This mode can be used when you have a real ECU paired and would like to compare the working of the fake ECU with a real one. If you go for this stealth mode, you also have to change the ID of the fake ECU. Make sure, the first 4 characters of the face ECU-ID is the same as for your real ECU. Make sure the rest of the fake ECU-ID is different from the real ECU ID.

If you have no real ECU do not change these parameters unless you know what you are doing.

How frequent data is collected. Normally, 5 min (as set per default) is a good for most users. Do not set this too low. Minimum value is 1 min.

Logging mode. 0: log string as received from inverters (raw). 1: log calculated values separated by “;”.  Use this if you want to export data received in Excel. 3: logs both calculated value and raw string (default).

3. License

This software is free for home use. Use in commercial applications is not allowed.

4. Disclaimer

Use it at your own risk. Tested only with YC600 inverters but might work with QS1 too. Data calculation and logging will not work  if you select mode 1 and 2 as the QS1 has 4 channels instead of 2 and data string is different. However, it might still work setting “interpretData” to 0 (raw mode logging).