Bilinear Auto Bed Leveling Details

Bilinear leveling is designed to compensate for a tilted bed and for peaks and valleys on the print bed.

Bilinear leveling algorithm

G29 creates a grid matrix of the Z compensation values at the probed points.

The planner calls the routine bilinear_z_offset to determine how much to offset the destination Z on all moves after G29 is run.

bilinear_z_offset does the following:

- Computes the indices for the four points in the grid matrix that surround the destination X.Y.

- Does a linear interpolation of the Z offset by doing the following:
2a. Draws a line between the left points in the grid and then calculates the Z for the destination Y.
2b. Draws a line between the right points in the grid and then calculates the Z for the destination Y.
2c. Draws a line between the calculated left & right points and then calculates the Z for the destination X. This is the Z offset returned to the planner.

With bilinear leveling the software stores the probe's location along with the Z compensation for that point. When printing the planner uses that same compensation when the nozzle is at that point.

Compensation areas:

If your probe is in front and to the right of your nozzle and you maximize the probed area then the compensation that’s applied will look like one of these.

The green area is where the probing was done.  Only points within the probed area are fully compensated.  Avoid printing outside that area.  To find the probed area do a G29 V4 command.  The min and max X & Y values are the edges of the probed area.

The other sections are where the probe couldn’t get to.

The blue corner area only applies the Z compensation from the corresponding corner in the grid. Essentially this area is treated as totally flat.

The yellow area has some compensation when moving in the Y direction but not when moving in the X direction.

The red area has some compensation when moving in the X direction but not when moving in the Y direction.

Main error contributors:

- Probe accuracy & repeatability.  M48 will give you a really good idea of the repeatability of the probe.  Some inductive probes have been known to change their trigger point as they came near the edge.

- Accuracy of the X & Y probe offsets.  If you have a 100mm bed that tilts 1mm, then a 5mm error in the X,Y offsets will give you a 0.05mm error in the compensated height.

- Accuracy of the X & Y nozzle offsets on multi-nozzle systems.   The effects are the same as with probe X,Y offset errors.

- Step size

- Mechanical hysteresis

- If you're within the probed area

- The density of the matrix – the more points that are probed the better the compensation